## Literature List (Note 2)

List of literature to be submitted with the offer.

Supporting documents and printed manufacturer’s technical literature furnished by the tenderer may be in another language, provided they are accompanied by an accurate translation into English. For the purposes of interpretation of the tender, the English language will prevail.

ALL BIDDERS ARE TO NOTE THAT PHOTOS SUBMITTED AS MANUFACTURER‘S TECHNICAL LITERATURE SHALL NOT SUFFICE AND ACCORDINGLY THESE MUST BE DULY ACCOMPANIED BY THE RESPECTIVE DETAILED MANUFACTURER‘S TECHNICAL LITERATURE.

The submission shall be in a structured form and is to be in the same sequence as listed hereunder for ease of reference and evaluation.

Economic operators are to declare that they would be in a position to provide the following works and documentation:

# Green Public Procurement (GPP)

|  | Paint Formulation: Section 4 – Technical Specifications, Clause 3.5.1 |
| --- | --- |
|  | White Pigment Content |
|  | The tenderer shall provide documentation for the paint formulation showing the content of white pigments. Where required, Class 1 wet scrub resistance shall be demonstrated based on a test report carried out according to EN 13300 using the method EN ISO 11998 (Test for cleanability and scrub resistance). Products which have been awarded the EU Ecolabel for paints and varnishes, as established in Commission Decision (EU) 2014/312/EU are deemed to comply. |
|  | Content of Volatile Organic Compounds |
|  | The tenderer shall provide either: - calculation of the VOC content, supported, if available, by Safety Data Sheets or - A test report carried out according to ISO 11890-2. Products with a VOC content lower than 1.0g/l shall be tested according to IS 17895 Products which have been awarded the EU Ecolabel for paints and varnishes, as established in Commission Decision (EU) 2014/312/EU are deemed to comply. |
|  | Product hazard labelling |
|  | The tenderer shall provide appropriate documentation confirming that the products to be supplied are not classified with the listed hazards. The documentation of the mixture classification shall be provided in accordance with the rules provided in Regulation (EC) No 1272/2008 (CLP Regulation) and/or Safety Data Sheets. Products which have been awarded the EU Ecolabel for paints and varnishes, as established in Commission Decision (EU) 2014/312/EU are deemed to comply. |
|  | Hazardous ingredients |
|  | The tenderer shall provide appropriate documentation confirming compliance with the criterion, namely: - for preservatives and APEOs: Safety Data Sheets for the product mixture. - for phthalates: Safety Data Sheets for the product mixture. - additionally for preservatives: a test report using OECD 305 Test Guideline can be used as an alternative to the Safety Data Sheet for the sole purpose of confirming that the preservatives used are non-bio-accumulative. - for formaldehyde: a test report based on the Merckoquant method or high-performance liquid chromatography (HPLC) method (See Annex 2), - for metals: a test report based on ISO 3856 series or equivalent. Products which have been awarded the EU Ecolabel for paints and varnishes, as established in Commission Decision (EU) 2014/312/EU are deemed to comply. |
|  | Spreading rate |
|  | The tender shall provide a test report using the following methods, or their equivalent: - ISO 6504/1 (Paints and varnishes — determination of hiding power — Part - 1: Kubelka-Munk method for white and light-coloured paints); - ISO 6504/3 (Part 3: determination of contrast ratio (opacity) of light-coloured paints at a fixed spreading rate); - NF T 30 073 for paints specially designed to give a three-dimensional decorative effect or which are characterised by a very thick coat. Products which have been awarded the EU Ecolabel for paints and varnishes, as established in Commission Decision (EU) 2014/312/EU are deemed to comply. |
|  | Weathering resistance (only outdoor paints) |
|  | The tenderer shall provide test results demonstrating performance of the paint according to the requirements listed in Table 8. With the exception of corrosion for metal paints the artificial weathering conditions shall reflect the conditions described in ISO 11507 or (for outdoor wood finishes) QUV accelerated weathering apparatus with cyclic exposure with UV(A) radiation and spraying according to EN 927-6 or their equivalent. For corrosion the relevant atmospheric corrosivity categories in EN ISO 12944-2 and the accompanying procedures specified in EN ISO 12944-6, or equivalent, shall be used. Anti-rust paints for steel substrates shall be tested after 240h salt spray following ISO 9227 or equivalent. Products which have been awarded the EU Ecolabel for paints and varnishes, as established in Commission Decision (EU) 2014/312/EU are deemed to comply with the above criteria. |
|  | Fungal and algal resistance of the film (only outdoor paints) |
|  | The tenderer shall provide test results demonstrating compliance according to the test methods EN 15457 and/ or EN 15458, or their equivalent. For coatings containing encapsulated dry-film biocides altered conditioning protocols shall also be accepted. Manufacturers shall provide information about any variation in conditioning along with test results of the EN 15457 and/or 15458 standards. Products which have been awarded the EU Ecolabel for paints and varnishes, as established in Commission Decision (EU) 2014/312/EU are deemed to comply |
|  | Management of waste and unused paint |
|  | The tenderer shall provide a documented waste management plan which shall include method statements for safe paint stripping, equipment cleaning and waste and unused paint handling and disposal, as well as the measures applied to minimise waste and unused paint |

|  | Toilets and Urinals: Technical Specifications, Clause 17.1.1 |
| --- | --- |
|  | Flushing Toilet Equipment: Full flush volume |
|  | Products holding a relevant Type 1 Eco-label fulfilling the listed requirements will be deemed to comply. Otherwise, test results according to the test procedure contained in standard EN 997 or equivalent standards shall be submitted. The testing shall be performed by laboratories that meet the general requirements of standard EN ISO 17025 or equivalent. |
|  | Flushing Toilet Equipment: Flushing system requirements |
|  | Products holding a relevant Type 1 Eco-label fulfilling the listed requirements will be deemed to comply. Otherwise, a test report showing that the flushing system of the product complies with the requirements of the respective EN standards mentioned above or equivalent standards will be accepted. |
|  | Flushing Toilet Equipment: Flushing Performance |
|  | Products holding a relevant Type 1 Eco-label fulfilling the listed requirements will be deemed to comply. Otherwise, a test report showing that the flush performance of the product complies with the requirements of Standard EN 997 or equivalent standard will be accepted. |
|  | Flushing Toilet Equipment: Installation instructions / user information |
|  | Products holding a relevant Type 1 Eco-label fulfilling the listed requirements will be deemed to comply. Other appropriate means of proof will also be accepted, such as written evidence from the manufacturer that the above clause will be met. |

|  | Sanitary Tapware: Technical Specifications, Clause 17.1.2 |
| --- | --- |
|  | Maximum available water flow rate |
|  | Products holding a relevant Type 1 Eco-label fulfilling the listed requirements will be deemed to comply. Otherwise, results of sanitary tapware testing according to the test procedure contained in the relevant EN standard (see the list in Table 2 below) or an equivalent standard shall be submitted together with the tender to the contracting authority. The testing shall be conducted at pressure of 1.5, 3.0 and 4.5 bar (± 0.2 bar) for products declared by the manufacturer as being suitable for high pressure installations (typically 1.0 to 5.0 bar) or at pressure of 0.2, 0.3 and 0.5 bar (± 0.02 bar) for products declared by the manufacture as being suitable for low pressure installations (typically 0.1 to 0.5 bar). The mean value of the three measurements shall not exceed the maximum water flow rate value indicated in Table 1. The testing shall be performed by laboratories that meet the general requirements of EN ISO 17025 or equivalent. A technical dossier from the manufacturer or other appropriate means of proof demonstrating that these requirements have been met will also be accepted. Table 2 EN standards for sanitary tapware; Number; Title. EN 200; Sanitary tap-ware. Single taps and combination of taps for water supply systems of type 1 and type 2 - General technical specification. EN 816; Sanitary tap-ware. Automatic shut-off valves (PN10). EN 817; Sanitary tap-ware. Mechanical mixing valves (PN10) - General technical specifications. EN 1111; Sanitary tap-ware. Automatic shut-off valves (PN10). EN 1112; Sanitary tap-ware. Shower outlets for sanitary tap-ware for water supply systems type 1 and type 2 - General technical specification. EN 1286; Sanitary tap-ware. Low pressure mechanical mixing valves. General technical specification. EN 1287; Sanitary tap-ware. Low pressure thermostatic mixing valves. General technical specification. EN 15091; Sanitary tap-ware. Electronic opening and closing sanitary tap-ware.. EN 248; Sanitary tap-ware. General specification for electrodeposited coatings of Ni-Cr. EN60335-1; Household and Similar Electrical Appliances. EN60335-2-35; Household and Similar Electrical Appliances, Safety, Particular Requirements for instantaneous water heaters. |
|  | Lowest maximum available water flow rate |
|  | Products holding a relevant Type 1 Eco-label fulfilling the listed requirements will be deemed to comply. Otherwise, result of sanitary tap-ware testing according to the test procedure contained in the relevant EN standard (see the list in Table 2) or an equivalent standard shall be submitted together with the tender to the contracting authority for verification. The testing shall be conducted at pressure of 1.5, 3.0 and 4.5 bar (± 0.2 bar) for products declared by the manufacture as being suitable for high pressure installations (typically 1.0 to 5.0 bar) or at pressure of 0.2, 0.3 and 0.5 bar (± 0.02 bar) for products declared by the manufacture as being suitable for low pressure installations (typically 0.1 to 0.5 bar). The mean value of the three measurements shall not be lower than the water flow rate value indicated in Table 3. The testing shall be performed by laboratories that meet the general requirements of EN ISO 17025 or equivalent. A technical dossier from the manufacturer or other appropriate means of proof demonstrating that these requirements have been met will also be accepted. |
|  | Temperature management |
|  | Products holding a relevant Type 1 Eco-label fulfilling the listed requirements will be deemed to comply. Other appropriate means of proof will also be accepted, e.g., manufacturer/supplier statement specifying the type of solution used and its technical parameters, as appropriate, shall be submitted. Where a water supply is already temperature controlled the tenderer shall explain the specific technical property that makes the sanitary tap-ware specifically designed to be fitted to this form of system. |
|  | Time control for sanitary tapware for multiple users and high frequency use |
|  | Products holding a relevant Type 1 Eco-label fulfilling the listed requirements will be deemed to comply. Other appropriate means of proof will also be accepted, e.g., manufacturer/supplier statement specifying the type of solution used and its technical parameters, as appropriate, shall be submitted. Where a water supply is already temperature controlled the tenderer shall explain the specific technical property that makes the sanitary tap-ware specifically designed to be fitted to this form of system. |
|  | Exposed surface condition and quality of coating |
|  | Products holding a relevant Type 1 Eco-label fulfilling the listed requirements will be deemed to comply. Otherwise, results of sanitary tap-ware testing according to the test procedure contained in the EN 248 standard or equivalent shall be submitted together with the tender to the contracting authority for verification. The testing shall be performed by laboratories that meet the general requirements of EN ISO 17025 or equivalent. A technical dossier from the manufacturer or other appropriate means of proof demonstrating that these requirements have been met will also be accepted. |
|  | Reparability and availability of spare parts |
|  | Products holding a relevant Type 1 Eco-label fulfilling the listed requirements will be deemed to comply. Other appropriate means of proof will also be accepted such as written evidence from the manufacturer that the above clause is met. The tenderer shall provide a description of how to replace components and provide a guarantee for the availability of spare parts. |
|  | User information |
|  | Products holding a relevant Type 1 Eco-label fulfilling the listed requirements will be deemed to comply. Other appropriate means of proof will also be accepted such as written evidence from the manufacturer that the above clause is met. |